Measuring White Rice Quality

What are the properties of milled rice that should be measured?

- Moisture,

- Grain dimensions

- 1000 grain weight

- Milling degree

- Milling recovery

- Dockage

- Head rice and broken grains

- Chalkiness

- Whiteness

- Grain shape

Measurements

4. Milling degree

Milling degree is computed based on the amount of bran removed from the brown rice. To obtain the weight of brown rice, dehull the paddy samples using the Laboratory Huller. Estimate the percent milling degree using the following equation:

% Milling degree = Wt of milled riceWt of brown rice x 100

5. Milling recovery

Using the Abrasive Whitener, mill the dehulled samples.  Compute milling recovery by dividing the weight of milled rice recovered by the weight of the paddy sample.

% Milling recovery = Wt milled riceWt of sample used x 100

6. Dockage

Select, segregate and weigh the foreign matter.  Record the number of unhulled grains collected from the sample.  Determine the percentage of dockage of milled rice using the equation:

% Dockage (mr) =Wt. ofdockageTotal wt of milled rice x 100

7. Head Rice and broken grain

Using the Grain Grader, separate the broken grain from the whole grains.  Compute the percentage of the milling recovery components using the following equations:

% Head rice =Wt of whole grainsWt of paddy samples x 100

# What are the properties of white rice that must be measured?

- Moisture,

- Grain dimensions

- 1000 grain weight

- Milling degree

- Milling recovery

- Dockage

- Head rice and broken grains

- Chalkiness

- Whiteness

- Grain shape

Measurements

8. Chalkiness

A visual rating of the chalky proportion of the grain is used to measure chalkiness based on the standard Evaluation System SES  scale presented below: Select, segregate and weigh the chalky grains (SES Scale 9).  Determine the % chalky grain using the equation

| % Chalky grain = Wt of chalky grainsWt of milled ricex100: Scale | % area of chalkiness |
| --- | --- |
| 1 | less than 10 |
| 5 | 10-20 |
| 9 | more than 20 |

9. Whiteness

- Measure the grain whiteness using the Whiteness Meter.

- % Yellow grains =Wt yellow grains Wt total milled rice x 100Separate and weigh yellow-fermented grains.  Calculate the percentage of yellow/fermented grains using the formula:

10. Grain Shape

| Scale | Shape | L/W ratio |
| --- | --- | --- |
| 1 | Slender | Over 3.0 |
| 3 | Medium | 2.1 – 3.0 |
| 5 | Bold | 1.1 – 2.0 |
| 9 | Round | 1.0 or less |

- Follow the procedure of determining grain shape of paddy.  Based on the length to width ratio, the shape of the milled rice will be determined. The ISO Classification is as follows:

LW ratio=Avg. length of riceAvg. width of rice x 100

% Brokens = Wt of broken grainsWt of paddy samples x 100

# For more information contact
Agricultural Engineering Unit
IRRI, DAPO Box 7777, Metro Manila, Philippines
Tel.: (63-2) 580-5600, Fax.: (63-2) 580-5699
Email: 	M.Gummert@cgiar.org
	J.Rickman@cgiar.org

IRRI  International Rice Research Institute